اثرهای زمان‏بندی پاشش مستقیم سوخت دیزل و نوع سوخت پیش‏آمیخته در یک موتور  RCCI

عباس عزتی1*، بهرام جعفری2

1- دانشجوی کارشناسی ارشد دانشکده فناوری نوین، دانشگاه تخصصی فناوری‏های نوین آمل، ezzati.abbas94@gmail.com

2- عضو هیأت علمی دانشکده فناوری نوین، دانشگاه تخصصی فناوری‏های نوین آمل،B.jafari@ausmt.ac.ir

چکیده

هدف اصلی این مقاله، ارزیابی اثرهای همزمان و جداگانه استفاده از متانول و بنزین به عنوان سوخت پیشآمیخته و زمانبندی تزریق مستقیم سوخت دیزل در یک موتور اشتعال تراکمی واکنش کنترل شده در شرایط بار سبک و سنگین است. نتایج نشان می‏دهد که در همه شرایط کارکردی و در تمام زمانبندیهای تزریق سوخت دیزل، فرآیند احتراق بنزین-دیزل در مقایسه با متانول-دیزل پایداری قابل ملاحظهای از خود نشان داده است. تزریق دیرهنگام سوخت دیزل منتهی به افزایش چشمگیر بیشینه فشار و دمای شعله در همه شرایط کارکردی موتور شده اما بر خلاف آن، در شرایط بار سبک در حالت بنزین-دیزل دمای شعله کاهش چشمگیری داشته است. کیفیت فرآیند اکسایش ترکیب سوخت-هوا برای حالت متانول-دیزل در همه شرایط کارکردی به ویژه بار سنگین بهتر بوده است. در حالت متانول-دیزل، تزریق دیرهنگام سوخت دیزل منتهی به تشکیل بیشتر اکسیدهای ازت و کاهش چشمگیر دوده در مقایسه با بنزین-دیزل شده است. تزریق زودهنگام سوخت دیزل در تمام شرایط کارکردی برای هر دو حالت سوخترسانی باعث کاهش قابل ملاحظه اکسیدهای ازت به نزدیکی صفر شده است. علاوه بر این، برای همه زمانبندیهای پاشش سوخت دیزل در تمام شرایط کارکردی موتور، بازده حرارتی متانول-دیزل در مقایسه با بنزین-دیزل کمتر گزارش شده است.

كليد واژه‏ها: شبیه‏سازی احتراق، موتور دوگانه‏سوز، آلایندگی، عملکرد.

Timing effects of direct injection of diesel fuel and type of premixed fuel in an RCCI engine

Abbas Ezzati1*, Bahram Jafari2

1- Master student of Modern Technology Department, Amol University of Special Modern Technologies, ezzati.abbas94@gmail.com

2- Faculty member of Modern Technology Department, Amol University of Special Modern Technologies, B.jafari@ausmt.ac.ir

Abstract

The main purpose of this article, Evaluation of simultaneous and separate effects of using methanol and gasoline as premixed fuels.And the timing of direct injection of diesel fuel in a compression ignition engine is a controlled reaction under light and heavy load conditions. The results show that in all operating conditions and in all diesel fuel injection schedules, the gasoline-diesel combustion process has shown considerable stability compared to methanol-diesel. Late injection of diesel fuel leads to a significant increase in maximum flame pressure and temperature in all engine operating conditions, but in contrast, in light load conditions in the gasoline-diesel mode, the flame temperature is significantly reduced. The quality of the oxidation process of the fuel-air combination for the methanol-diesel mode has been better in all operating conditions, especially heavy loads. In the methanol-diesel mode, late injection of diesel fuel has resulted in the formation of more nitrogen oxides and a significant reduction in soot compared to gasoline-diesel. Early injection of diesel fuel in all operating conditions for both refueling modes significantly reduced nitrogen oxides to near zero. In addition, for all diesel fuel spraying schedules in all engine operating conditions, the thermal efficiency of methanol-diesel has been reported to be lower than that of gasoline-diesel.

Keywords: Combustion simulation, dual combustion engine, pollution, performance.