## Design, manufacture and evaluation of cooling exhaust manifold

## Adib Souzani1*, Arash Mohammadi

| 1* MSc Student, Biosystem Mechanical Engineering, Tarbiat Modares University | Adib.souzani@modares.ac.ir |
| --- | --- |
| 2 Assistance professor, Shahid Rajaee Teacher Training University | amohammadi@sru.ac.ir |

## Abstract

## One of the applications of combustion engines is the use of these engines in vessels, which can be used both inboard and outboard engine, which unfortunately in the conditions of sanctions is not possible to buy this category of engines in the country. Taking into account the country's need for these engines and examining domestic facilities and capabilities, research and development of these engines is underway. According to the studies conducted in this field and considering all aspects, it can be said that the cheapest way is to use a car engine and redesign it as an marine inboard engine  installation. One of the most important parts that needs to be changed is the exhaust manifold, the performance of which has a significant impact on engine efficiency. Proper estimation of heat transfer and temperature distribution in the engine is essential to investigate thermal stresses. With the rapid advancement of computer technology and numerical methods, computer simulation has become a useful tool for research and development of fluid flow systems. In this study, computational fluid dynamics (Fluent) software was used for exhaust manifold analysis. A number of calculations were performed to investigate the parametric effects of operating conditions and geometry on the performance of the manifold, and detailed information on the flow distribution and heat transfer properties was obtained to improve a basic understanding of its performance. It was observed that with increasing the inlet temperature of the gas, the temperature of the outer shell of the manifold increases by 10 degrees Celsius, and this indicates that two multi-wall walls, regardless of the inlet gas temperature, can be very effective in preventing heat dissipation to the engine room. Be. Finally, suggestions for improving the design and performance of the cooling manifold are provided.

## Keywords: Marine engine, cooling passage, exhaust manifold, computational fluid dynamics