بررسی تاثیر پارامترهای هندسی و سرعت دیواره بر توزیع نانوذرات در یک دستگاه هم زن

نام و نام‌خانوادگی میثم یحیایی، مصطفی ورمزیار

1- دانشجوی کارشناسی، دانشکده مهندسی مکانیک، دانشگاه تربیت دبیر شهید رجایی، تهران

2- استادیار، مهندسی مکانیک، دانشگاه تربیت دبیر شهید رجائی، تهران

* تهران، صندوق پستی 16785-136، m.yahyaei1994@Gmail.com

چکیده

در دههی گذشته، با توجه به افزایش کاربردهای نانوسیالات و وسیع شدن طیف کاربردهای آن در زمینههای مختلف، تحقیقات و مطالعات بر روی نانوسیالات به سرعت افزایش یافته است. برای تولید نانوسیالات دو روش، شامل روش تک مرحلهای و دو مرحلهای وجود دارد. در روش دو مرحلهای، نانوذرات که به صورت جداگانه تولید شدهاند باید با سیال ترکیب شود و نانوسیال نهایی کاملا همگن باشد. هدف از این پژوهش، بررسی چگونگی تهیهی نانوسیال همگن حاوی ذرات آهناکسید (Fe3O4) در نوعی روغن بود. بدین منظور از یک همزمن مکانیکی بهنام دستگاه کوئت استفاده شد. شرط همگن شدن بر پایهی رابطهای از غلظت تعریف شد. همچنین، تاثیر ابعاد دستگاه کوئت و سرعت دوران بر زمان رسیدن به مخلوط همگن بررسی شد. نتایج نشان داد که با ا افزایش سرعت دوران، زمان دستیابی به مخلوط همگن افزایش مییابد. همچنین مشاهده شد که با زیاد کردن نسبت شعاع استوانههای بیرونی و درونی، زمان همگن شدن کاهش مییابد. در بخش آخر، نتیجه گرفته شد که حرکت نوسانی استوانهی درونی، بر زمان رسیدن به مخلوط همگن تأثیر منفی دارد.

کلید‌واژگان

نانوسیال ، نانوذرات، دستگاه کوئت، کامسول

Study of the effects of geometrical parameters and wall velocity on the distribution of nanoparticles in a blender

Meysam Yahyaei, Mostafa Varmazyar

1- Undergraduate Student, Department of Mechanical Engineering, Shahid Rajaee Teacher Training University, Tehran, Iran.

2- Assisstant profesor, Department of Mechanical Engineering, Shahid Rajaee Teacher Training University, Tehran, Iran

* P.O.B. 16785-136 Tehran, Iran, M.Yahyaei1994@Gmail.com

Abstract

In the past decade, due to the increasing use of nanofluids and expand their range of applications in various fields, researches and studies on nanofluids have grown rapidly. There are two methods for producing nanofluids, including single-step and two-step methods. In the two-step method, nanoparticles have produced separately, It should be combined with fluid, and the final nanofluid should be completely homogeneous. This study aimed to investigate how to prepare homogeneous nanofluid containing Fe3O4 particles in a kind of oil. For this purpose, a mechanical synchronizer called the Couette device was used. The homogeneity condition was defined based on a relation of concentration. Also, the influence of the dimensions of the Couette device and the rotational velocity on the time to reach the homogeneous mixture was investigated. The results showed that by increasing of velocity of rotation, the time for achieving the homogenous mixture is reduced. Also, it is observed that by increasing the ratio of the outer and inner radius of cylinders, the time of homogenization is reduced. In the last section, it is concluded that the oscillatory motion of the inner cylinder had a negative impact on achieving a homogeneous mixture.

Keywords

Nanofluid, nanoparticles, couette device, comsol

. نویسنده مسئول