تاثیر طول فرول روی توزیع تنش‌ در ترمیم‌های پست کور کراون به کمک روش اجزای محدود

مجتبی محمودی1، پرویز امینی*2، محمدرضا فرات یزدی3

1- استادیار، مهندسی مکانیک، مجتمع آموزش عالی بم، بم

2- دانشیار، پروتزها دندانی، دانشگاه علوم پزشکی کرمان، کرمان

3- فارغ التحصیل دکترای حرفه‌ای، دندانپزشکی عمومی، دانشگاه علوم پزشکی کرمان، کرمان

* کرمان، صندوق پستی 7618759689، p_amini@kmu.ac.ir

چکیده

يكي از روش‌هايي كه براي افزايش مقاومت دندان‌ها در برابر فشارهاي اكلوزالي به كار مي‌رود استفاده از اثر فرول مي‌باشد. هدف از مطالعه حاضر تعیین اثر فرو‌ل بر مقاومت به شكست دندان‌هاي بازسازي شده با پست‌هاي فايبرگلاس به کمک روش اجزای محدود می‌باشد. یک دندان نیش با شکل و ابعاد آناتومی نرمال انتخاب و درون یک رزین مانت شد. سپس یک اسکن سه بعدی از دندان مانت شده تهیه شد و به کمک نرم افزارهای میمیکس، و سالیدورکس مدل سه بعدی و دقیقی از آن تهیه شد. ترمیم پست کور کراون با سه طول مختلف برای فرول (1، 5/1و 2 میلیمتر) لحاظ و سپس آنالیز اجزای محدود به کمک نرم افزار آباکوس انجام ‌شد. آنالیز اجزای محدود روی مدل‌ها تحت یک بار توزیع شده به شدت 100 نیوتن با زاویه 45 درجه نسبت به محور طولی انجام شد تا اثر طول فرول روی تمرکز تنش‌ها بررسی شود. نتایج اجزای محدود نشان داد که در تمام حالت‌ها حداکثر تنش اصلی در عاج و در لبه کرست رخ  می‌دهد (52-50 مگاپاسکال). با افزایش طول فرول تنش در عاج کمی افزایش و در ناحیه سرویکال کاهش می‌یابد. طبق نتایج اجزای محدود مشخص شد که افزایش طول فرول باعث کاهش تنش در سمان کراون خصوصا در ناحیه سرویکال و در نتیجه منجر به افزایش مقاومت به شکست دندان ترمیم شده می‌شود.

کلید‌واژگان

اجزای محدود، تنش، فرول، پست-کور

Influence of Ferrule Length on Stress Distribution at Post-Core Crown Restorations by Finite Element Method

Mojtaba Mahmoudi1, Parviz Amini2*, Mohammad Reza Forat Yazdi3

1- Department of Mechanical Engineering, Higher Education Complex of Bam, Bam, Iran

2- Department of Prosthodontics, Dental College, Kerman University of Medical Science, Kerman, Iran

3- Department of Dental College, Kerman University of Medical Science, Kerman, Iran

* P.O.B. 7618759689 Kerman, Iran, p_amini@kmu.ac.ir

Abstract

One of the methods which is used to increase the teeth resistance, verses occlusal loads is the use of ferrule effect. The aim of this study was to determine the effect of ferrule on the fracture resistance of restored teeth with glass fiber posts using finite element method. A canine tooth with a normal anatomical shape and dimensions was chosen and mounted inside a resin. Then, a 3D scan of the mounted tooth was prepared and an exact three-dimensional model was provided by Mimics and Solidworks softwares. Post-core crown restoration with three different length (1,1.5, and 2 mm) was considered for ferrule and then the finite element analysis was performed by ABAQUS software. The finite element analysis was done under a distributed load of 100 N at 45-degree angle to the longitudinal axis to investigate the effect of ferrule on stress concentration. The results of finite element analysis showed that the maximum principle stress occurred in dentin and at the crest edge in all states (50-52 MPa). Stress increased slightly in dentin and decreased in the cervical region as the ferrule length increased. According to the finite element results, the stress at the crown cement was reduced by increasing the ferrule height therefore it can be concluded that the fracture increase strength of restored tooth as the ferrule height increase.

Keywords

Finite element, Stress, Ferrule, Post-core