بروز رسانی مدل دینامیکی سازه بال نوعی پهپاد با استفاده از  الگوریتم ژنتیک و در محیط نرمافزار Ansys

احمد کریمی1، محمد رحیم زاده2*

۱- کارشناسی مهندسی مکانیک، دانشکده مهندسی، دانشگاه گلستان، گرگان، ایران

۲- استادیار، گروه مهندسی مکانیک، دانشکده مهندسی، دانشگاه گلستان، گرگان، ایران

*نویسنده مسئول:  m.rahimzadeh@gu.ac.ir

چکیده

با ظهور کامپیوترهای قدرتمند بررسی رفتار دینامیکی سازههای مکانیکی به روش اجزاء محدود بخوبی امکانپذیر شده است. اما همواره بدلیل وجود خطا در مدل اجزاء محدود از قبیل خطای گسسته سازی، عدم مدلسازی دقیق شرایط مرزی و اتصالات، خواص مواد و غیره، عدم انطباقی بین نتایج آزمایشگاهی و مدل اجزاء محدود وجود دارد. این امر بیانگر لزوم بهبود مدل اجزاء محدود اولیه است. این تحقیق بر بروز رسانی مدل اجزاء محدود سازه بال یک پهپاد با ایرفویلهای NACA-2412 متمرکز است. فرایند بروز رسانی در  نرمافزار تجاری Ansys و با ابزارهای بهینهسازی آن انجام شده است. در این روش ابتدا با اعمال ۵٪ خطا در مقادیر چگالی و مدول الاستیسیته ایرفویلها، نتایج آزمایشگاهی شبیهسازی شدهاند. مقادیر فرکانسهای طبیعی و شکل مودهای حاصل از این تغییر، به عنوان تابع هدف در بروز رسانی مدل درنظر گرفته شد. با اعمال فرایند بروز رسانی مشاهده گردید که معیارهای همبستگی مبتنی بر فرکانسهای طبیعی و شکل مودها، حکایت از هماهنگی خوب مدل بروز شده با مدل آزمایشگاهی دارد.

کلید‌واژگان

بروز رسانی مدل دینامیکی، اجزاء محدود، بهینهسازی، الگوریتم ژنتیک، Ansys

Dynamic model updating of a kind of UAV wing structure using a genetic algoritm in the Ansys software

Ahmad Karimi1, Mohammad Rahimzadeh2*

1- BSc., Department of Mechanical Engineering, Golestan university, Gorgan, Iran

2- Department of Mechanical Engineering, Faculty of Engineering, Golestan university, Gorgan, Iran

Email: m.rahimzadeh@gu.ac.ir

Abstract

With the advent of powerful computers, it has become possible to study the dynamic behavior of mechanical structures by the finite element method. However, due to errors in the finite element model, such as discretization error, inaccurate modeling of boundary conditions and joints, material properties, etc., there is a mismatch between the experimental results and the finite element model. This indicates the need to improve the primary finite element model. This research focuses on updating the finite element model of the wing structure of a UAV with NACA-2412 airfoils. The update process is done in Ansys commercial software and with its optimization tools. In this method, experimental results are first simulated by applying a 5% error in the density values ​​and elasticity modulus of the airfoils. The values ​​of the natural frequencies and the mode shapes from this change were considered as the objective function in updating the model. By applying the updating process, it was observed that the correlation criteria based on natural frequencies and mode shapes indicate good corrolation of the updated model with the experimental model.

Keywords

Dynamic model updating, Finite Element method, optimization, Genetic Algoritm, Ansys