شبیه سازی عددی انتقال حرارت اثر سطوح ثانویه روی جریان جت نوسانی برخوردی به سطح

بابک یوسفی لفورکی1*، مهران رجبی زرگرآبادی 2

1- دانشجوی دکتری، دانشکده مهندسی مکانیک، دانشگاه سمنان، سمنان

2- دانشیار، دانشکده مهندسی مکانیک، دانشگاه سمنان، سمنان

* سمنان، صندوق پستی 35131-191، babak86yousefi@gmail.com

چکیده

بررسی عددی انتقال حرارت آشفته در یک جت  دایروی نوسانی برخوردی به سطح صاف دارای پین-فین انجام شد. در این مقاله از پین –فین ها به عنوان سطوح ثانویه مورد استفاده قرار گرفت. پین – فین ها یکی از پرکاربردترین فن‌آوری‌های خنک‌کننده در لبه پیشانی پره های توربین گاز می باشند. در این راستا معادلات متوسط گیری شده برای جریان تراکم ناپذیر آشفته در حالت غیر دایم به همراه مدل آشفتگیRNG  k-𝛆  در یک فضای محاسباتی سه بعدی حل شده‌اند. تاثیر فرکانس نوسان، فاصله شعاعی پین از جت و عدد رینولدز جت بر توزیع متوسط زمانی عدد ناسلت سطح مورد بررسی قرار گرفته است. نتایج به دست آمده نشان می‌دهد که در مقایسه با جت پایا، استفاده از جت نوسانی در محدوده فرکانس 50 تا  100 هرتز باعث افزایش میانگین انتقال حرارت از سطح می‌شود. این در حالی است که با افزایش فرکانس، افزایش قابل توجهی برای عدد ناسلت در روی سطح مشاهده نشده است. همچنین با افزایش سطح جانبی پین متوسط زمانی عدد ناسلت افزایش می‌یابد. مشهود است که بهترین مکان برای قرار گرفتن پین روی سطح به فاصله سه برابر قطر جت از مرکز جت (r=3D) می‌باشد.

کلید‌واژگان

پین-فین، جریان نوسانی جت برخوردی، عدد ناسلت، انتقال حرارت

Numerical simulation of the effect of secondary surface on heat transfer from pulsating impinging jet

Babak Yousefi-lafouraki1*, M. Rajabi-Zargarabadi 2

1- PhD Stu, Mech. Eng., Semnan Univ., Semnan, Iran

2- Assoc. Prof., Mech. Eng., Semnan Univ., Semnan, Iran

* P.O. B. 35131-191, Semnan, Iran, babak86yousefi@gmail.com

Abstract

Numerical simulation was performed to investigate the turbulent heat transfer characteristics of a circular pulsating jet impinging on a flat plate mounted pin-fin. In this study, Pin-fins has been used as a secondary surface. Pin-fins are one of the most widely used cooling technology in turbine blade trailing edge. In this way, the averaged Navier-Stokes equations for turbulent incompressible flow in an unsteady state with k-ε RNG turbulence model and in 3D computational space were solved. The effects of the oscillation frequency, radial distance pin to jet and jet Reynolds number on time- averaged Nusselt number distribution in the asymmetrical concave surface were studied. The obtained results show that applying the pulsating jet in the range of 50 Hz to 100 Hz can increase heat transfer from the surface in comparison with the steady jet. This is while by increasing the frequency no significant increase for the Nusselt number at the impinging surface has been observed. Furthermore, increasing area of pin leads to the increase of the time-averaged Nusselt number. It is found that the best place to locate the pin on the surface is three times the diameter of the jet from the center of the jet (r = 3D).

Keywords

Pin-fin, pulsating impinging jet, Nusselt number, Heat transfer