مطالعه پارامتری سیکل کالینا در یک نیروگاه زمین گرمایی به روش تراکم فشاری پیشرونده

معین چراغیزاده1، محمد افتخاری یزدی2*، غلامرضا صالحی2

1- دانشجوی کارشناسی ارشد مهندسی مکانیک گرایش تبدیل انرژی، دانشگله آزاد اسلامی واحد تهران مرکز، تهران، ایران

2- استادیار، مهندسی مکانیک، دانشگاه آزاد اسلامی واحد تهران مرکز، تهران

* تهران، صندوق پستی 999999999، moh.eftekhari-yazdi@iauctb.ac.ir

چکیده

امروزه بخش عمده برق جهان را سوختهای فسیلی تأمین میکنند. با در نظر گرفتن محدودیتهای مربوط به سوختهای فسیلی و مشکلات زیست محیطی آنها، طراحی سیستمهای تولید توان با بازدهی بالا یک ضرورت محسوب میشود. در این مقاله هدف تحلیل ترمودینامیکی سیکل کالینا با روش تراکم فشاری پیشرونده میباشد که از دیدگاه قانون اول بررسی میشود. ابتدا عملکرد مبتنی بر تحلیل انرژی یک چرخه کالینا بررسی شده و نتایج حاصل با نتایج موجود در ادبیات فن اعتباردهی میشود. مدلسازی سیستمها با استفاده از نرمافزار EES  انجام خواهد گرفت که یک نرم افزار مناسب جهت حل معادلات ترمودینامیکی است. علاوه بر آن ارائه یک برنامه کامپیوتری که میتواند در شرایط محیطی و عملیاتی مختلف خروجیهای سیستم را نشان داده و نقاط بهینه عملکرد سیستم را مشخص نماید، از بخشهای حائز اهمیت نوآورانه محسوب میگردد. با ثابت ماندن مقدار حرارت دریافتی از محفظه احتراق، مجموع توان خالص تولیدی، بار سرمایشی و گرمایشی سیستم با افزایش دمای مبدل حرارتی ۱ کاهش مییابد که نتیجه آن کاهش بازده انرژی از ۹۴ به ۸۹ درصد میباشد. به علاوه چون مجموع توان تولیدی و اگزرژی سرمایشی و گرمایشی با افزایش دمای مبدل حرارتی ۱ افزایشی از ۱۰ الی ۱۱ درصد مییابد و بازده اگزرژی نیز آهنگ افزایشی دارد.

کلید‌واژگان

انرژی، قانون اول ترمودینامیک، کالینا، اگزرژی، آمونیاک

Parametric study of Kalina cycle in a geothermal power plant by sliding

condensation pressure method

Moein Cheraghizadeh1, Mohammad Eftekhari Yazdi2*, Gholamreza Salehi2

1- Master student of Mechanical Engineering, Department of Mechanical Engineering, Central Tehran Branch, Islamic Azad University, Tehran, Iran

2- Assistant Professor of Mechanical Engineering, Department of Mechanical Engineering, Central Tehran Branch, Islamic Azad University, Tehran, Iran

* P.O.B. 999999999 Tehran, Iran, moh.eftekhari-yazdi@iauctb.ac.ir

Abstract

Today, most of the world's electricity is supplied by fossil fuels. Given the limitations of fossil fuels and their environmental problems, designing high-efficiency power generation systems is a necessity. In this paper, the purpose of thermodynamic analysis of Kalina cycle with progressive compression compression method, which is examined from the perspective of the first law. First, the performance based on energy analysis of a Kalina cycle is investigated and the results are validated with the results in the technical literature. Systems modeling will be done using EES software, which is a suitable software for solving thermodynamic equations. In addition, providing a computer program that can show the system outputs in different environmental and operational conditions and determine the optimal points of system performance, is one of the most important parts of innovative. By keeping the amount of heat received from the combustion chamber constant, the total net power output, cooling and heating load of the system decreases with increasing the temperature of the heat exchanger 1, which results in a reduction in energy efficiency from 94 to 89%. In addition, because the total production capacity and cooling and heating exergy increases by 10 to 11% with increasing the temperature of the heat exchanger 1, and the exergy efficiency also increases.

Keywords

Energy, the first law of thermodynamics, Kalina, exergy, ammonia