| تبیین عملکرد صوتی سردر قیصریه نقش جهان سیما سعادتمند، بهزاد وثیق - کارشناس ارشد معماری دانشگاه صنعتی جندی شاپور دزفول - استادیار گروه معماری دانشگاه صنعتی جندی شاپور دزفول |  |
| --- | --- |
| explanation of the acoustic performance of Qeysariye Gate of Naghsh e Jahan Sima Saadatmand , Behzad Vasiq - Master of architecture Jundi-Shapur University of Technology,Dezful,Iran - Assistant professor,Jundi-Shapur University of Technology,Dezful,Iran |  |
| *Corresponding Author: saadatmandsima94@gmail.com saadatmandsima94@gmail.com: ایمیل نویسنده مسئول |  |
| چکیده |  |

تا کنون ارزش فضاهای معماری تنها بر اساس سنجش تزیینات و کالبد بیرونی آن بررسی شده است و به لایههای دیگر مانند کارکرد اقلیمی یا کارکرد منظری آن توجه

کمتری شده است.همچنین در زمینه منظر عمدتاً به فضاهای باغ یا فضاهایی که با محوریت سبز هستند توجه شده و از منظر صوتی یا بویایی که در فضاهای معماری وجود دارد اشاره نشده است.در این مقاله تلاش میشودتا با معرفی سردر قیصریه در مجموعه نقش جهان اصفهان به اثر شکلی آن بر روی کاهش نوفهی محیطی پرداخته شود.روش تحقیق در این مقاله مورد کاوی و روش پیمایشی است.گردآوری اطلاعات براساس مطالعات میدانی و همچنین اسنادی صورت گرفته است.روش آزمون تحقیق به این صورت است که پلانهای رقیب جهت مقایسه کارکرد صوتی آن نسبت به پلان مادر (وضع موجود سردر) طراحی شده و براساس شعاعهای ساطع شده صدا بین آنها مقایسه میشود.فرض بر آن است که هر مقدار که یک کالبد بتواند پرتوهای ساطع شده صدا را بیشتر دچار شکست کند، موجب میشود که در میزان شدت صوت کاهش بوجود آمده و منظر صوتی مطلوبی به ما بدهد.در بررسی مشخص شد که این مجموعه به علت نوع عقبرفتگی ورودی ،میزان عقب رفتگی طاقچهها ،دو راسته بازاری که در طرفین قرار دارند و نیز مقرنسی که در سردر قیصریه وجود دارد باعث شده است که منظر صوتی دارای کیفیت بهتری باشد.

واژههاي کليدي

آسایش صوتی، بازار ، سردر قیصریه

Abstract

So far, the value of architectural spaces has only been assessed based on its exterior decoration and body, and less attention has been paid to other layers such as its climatic or landscape function. Landscaping has also focused mainly on garden areas or green-focused areas, and does not address the acoustic or olfactory aspects of architectural spaces. In this paper, it is attempting to introduce its effect on environmental noise reduction by introducing Gheisariyeh in the Naqsh e Jahan of Isfahan. The research method in this article is a case study and survey method. The data collection was based on field studies as well as documents. The test method of the research is that the competing plans are designed to compare its acoustic performance with that of the motherboard (present status of the gate) and are compared between them based on the radiated sound. It is assumed that any amount that a body can more successfully defeat the emitted sound will cause a decrease in the amount of sound intensity and give us a good sound perspective. The study revealed that this set has a better sound quality due to the type of input retardation, the amount of niche retardation, the two market orders that are on either side, as well as the Mogharnas present in the Qaysariyeh gate.

Keywords:

Acoustic comfort , Bazaar, Qaysariyeh Gate